Specifications text

Article number

222184

Type

IPP-62A0020M0B

Short description

1/2,8" Network Dome, PTZ, Day&Night, 20x, H.265, WDR, 1920x1080, PoE, IP66

Main Features

1/2.8" Sony STARVIS CMOS-Sensor

Full HD 1920x1080 Pixel, 25/30fps

23x AF Zoom, 4.7~94mm

Quad Stream H.265, H.264, MJPEG (Smart Codec)

Real time WDR (Wide Dynamic Range) 25/30fps

Intelligent motion detector

Digital Noise Reduction (2DNR/3DNR)

Audio support including audio alarm

Alarm in- and outputs, micro SD-card slot

Power supply 12VDC, 24VAC or PoE

IK10 IK shock resistance degree

Support ONVIF Profile S und Profile T

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 150x154mm |
| Series | eneo SN-Series |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,2 Lux at F1,6, 50 IRE |
| Digital Noise Reduction | yes |
| Lens Type | auto focus zoom |
| Focal Length | 4,7 - 94 mm |
| Iris Control | DC |
| Zoom Factor | 20x |
| Digital Zoom | 16x |
| Observe (62 px/m) | 28 - 550 m |
| Recognize (125 px/m) | 13 - 270 m |
| Identify (250 px/m) | 7 - 130 m |
| Detect (25px/m) | 70 - 1370 m |
| Horizontal Angle of View | 55,5 - 3° |
| Vertical Angle of View | 32,4 - 1,7° |
| MOD (Mininum Object Distance) | 1 m |
| llumination | no |
| Housing | Indoor, Outdoor |
| Color (housing) | white |
| Color (bubble) | clear |
| Hosuing material | Aluminium |
| Heater | optional |
| Window heater | optional |
| Blower | yes |
| Rotation range | 360° |
| Speed (manual) | 380°/Sek. |
| Speed (preset) | 380°/Sek. |
| Video outputs | no |
| Alarm inputs | 4 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSDHC card, microSDXC card |
| Analysis functions | Defog, Digital image stabilization, Motion detection, Perimeter Intrusion Detection (PID), Tampering Detection (TD) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, 24VAC, PoE |
| Power consumption | 12W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | yes |
| Operating temperature | -20°C ~ +50°C |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Certifications | IK10 |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | PTZ |
| Audio | 2 Way Audio |
| Remote configuration | Browser |
| Weight | 1.6 kg |
| Manufacturer order number | 222184 |

Additional Productinformation

Related products

| DOME-WB2-W | Wall bracket for IED-63xx Metal series and IEP-63Mxx, MEP-62Mxx |
| --- | --- |
| EDMC/CWB-1-W | Wall or Pendant Mount Kit white, for Dome Camera Series MPP-72xx, IPP-72xx |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| WDD064G1P0C | MicroSDXC Speicherkarte, 64GB, Class 10, UHS-I U1, 32TBW, für Videoüberwachung |
| --- | --- |

Services

| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| --- | --- |
| LACKIERUNG PTZ oder Panoramakamera | Extra charge for special paint PTZ dome or panoramic camera in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |