Specifications text

Article number

217917

Type

VM-FHD43M

Short description

43" (109cm) LCD Monitor FHD, 1920x1080, LED, HDMI, VGA Composite, Metal cabinet

Main Features

Professional 43" (109 cm) LCD monitor

Industrial design built to run 24x7x365 continuously

Maximum resolution 1920x1080 pixels

LED Back Light

Fast response time 5ms

Perfect presentation through digital 3D filtering and noise reduction

Different video inputs HDMI, VGA, Composite

Built-in speakers

Supply voltage: 240-100VAC, 50/60Hz

Metal cabinet including stand

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Refresh rate | 60 Hz |
| Dimensions (HxWxD) | 576,5x972,1x100,1mm |
| Series | eneo Monitor |
| Applications | 24/7 |
| Resolution Pixels | 1920x1080 |
| Housing color | black |
| Hosuing material | Metal |
| Backlighting | LED |
| Screen size (inch) | 43" |
| Screen size (cm) | 109 cm |
| Aspect ratio | 16:9 |
| Viewing angle vertical | 178° |
| Viewing angle horzontal | 178° |
| Brightness typical | 350 cd/m² |
| Contrast ratio typical | 3500:1 |
| Reponse time typical | 5 ms |
| Video inputs (type) | 2x composite (CVBS), HDMI, VGA |
| Loudspeaker | 2 |
| Video outputs | Composite (CVBS) |
| Audio inputs | 1 |
| Serial interfaces | no |
| Supply voltage | 120~230VAC |
| Power consumption | 80W |
| Operating temperature | -30°C ~ +85°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 25kg |
| Audio output | 1x 3,5mm jack |
| Manufacturer order number | 217917 |

Additional Productinformation

Related products

| VM-LCDWMB1 | Universal Wall Mount for LCD Monitors, 32" to 65", VESA support up to 35Kg max. |
| --- | --- |

Recommendations

| jetcon-HSA-HSA-1 | Economy HDMI Kabel Stecker A - Stecker A, 1m, HDMI High Speed mit Ethernet |
| --- | --- |
| jetcon-HSA-HSA-10B | HighSpeed HDMI Kabel mit Ethernet, 4K 60Hz, A-A Stecker 10m, schwarz |
| jetcon-HSA-HSA-3 | Economy HDMI Kabel Stecker A - Stecker A, 3m, HDMI High Speed mit Ethernet |

Services