Specifications text

Article number

220196

Type

ISD-59P0010P0A

Short description

1/1,7" Network Dome, fixed, Day/Night, 4000x3000, H.265, 1,29mm, 360°, infrared, IP66

Main Features

1/1.7" Sony STARVIS CMOS-Sensor

1.29mm fixed lens FishEye camera

360° Panorama-, Corridor-, Quad-, Panomorph-View, ceiling or wall

Integration in eneo Center Software

True Day & Night

Resolution max. 4000x3000@20fps

Quad Stream (H.264/H.265x3, MJPEGx1)

Cross Web-Browsing

True-WDR (Digital Wide Dynamic Range)

Framerate increase on motion detection (Smart Codec)

IR-LED Illumination integrated

Compact design for indoor application

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 58,7x151,4mm |
| Series | eneo SN-Series |
| Camera Type | Fisheye Kamera |
| Resolution Standard | 8 Megapixel |
| System | True Day&Night |
| Sensor size | 1/1,7" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 4000x3000 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,3 Lux at F2,8 |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 1,29 mm |
| Iris Control | DC |
| Digital Zoom | yes |
| Observe (62 px/m) | 25 m |
| Recognize (125 px/m) | 11 m |
| Identify (250 px/m) | 5 m |
| Detect (25px/m) | 60 m |
| Horizontal Angle of View | 360° |
| MOD (Mininum Object Distance) | 0,1 m |
| llumination | Infrared |
| Color (housing) | white |
| Hosuing material | Metal |
| Heater | no |
| Blower | no |
| Alarm inputs | 1 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD card, microSDXC card |
| Analysis functions | Defog, Intrusion, Loitering detection, Motion analysis, Motion detection, Person recognition, Tampering protection, Trip wire plus object counting |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 12.5W @ PoE,, 15W @ VDC12V |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | yes |
| Operating temperature | -20°C ~ +50°C |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Certifications | IK10 |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | Dome |
| Audio | 2 Way Audio |
| Remote configuration | Browser |
| Illumination range | approx. 10m |
| Weight | 750g |
| Manufacturer order number | 220196 |

Additional Productinformation

Related products

| AK-8B | Camera junction box, Wall and Ceiling mounting |
| --- | --- |
| AK-9B | Camera junction box for DH-6, Dome-WB4 |
| eneo Site Manager | Management Software for eneo IP products |
| IAD-5SG1008MUA | Gigabit Switch, Unmanaged, 120W, 8x PoE, 1xSFP, 1x RJ45, 10/100/1000 Mbps, Desktop |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |

Services

| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| --- | --- |
| LACKIERUNG PTZ oder Panoramakamera | Extra charge for special paint PTZ dome or panoramic camera in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |