Specifications text

Article number

43274

Type

F03Z2.6DC-NFSHR

Short description

F1.0/3-8mm DC Lens, Varifocal, No Focus Shift, High Resolution, 1/3" CS Mount

Main Features

Suitable for 1/3" and 1/4" day&night cameras

Variable focal length/Fast maximum aperture (F1.0)

High resolution realizing 650 TV lines

Clear images without colour smear

Minimized chromatic abberation

No-focus-shift between visible and near IR light

Aspherical lens elements

LD (Low Dispersion) glass

CS mount

Without built-in auto iris amplifier

With 4 pin connector

Adjustable slip-ring mount for cable positioning

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | see drawing |
| Diameter | 43,2 mm |
| Length | 54,4 mm |
| Lens Type | Varifocal |
| Day&Night | yes |
| Megapixels | no |
| Mount | CS |
| Nominal Format | 1/3" |
| Focal Length | 3 - 8 mm |
| Iris Control | DC |
| Horizontal Angle of View | 77,4 - 33,4° |
| MOD (Mininum Object Distance) | 0,3 m |
| Color | anthracite |
| Operating temperature | -20°C ~ +60°C |
| Aperture range (F) | F1.0~F360 |
| Weight | 75g |
| Manufacturer order number | 43274 |

Additional Productinformation

Related products

Recommendations

Services