Specifications text

Article number

226845

Type

SOL-IQbox-plus-B

Short description

camera connection box for video security with lightning protection, 400x300x210mm

Main Features

Optimised for outdoor use

Electrically prewired for flexible, modular expansion.

UV stable polycarbonate, corrosion-resistant

Protective class IP66

Impact resistance IK 10, vandalism protected

Swivelling door (with door contact switch)

Wall or mast mounted

Fast, easy installation

Simple mechanical finishing

Including service socket and electrical circuit breaker

Lightning protection combination arrestor type

Other Features (Specifications)

| Brand | Solvido |
| --- | --- |
| Series | Solvido IQbox |
| Housing | Outdoor |
| Color (housing) | RAL7035 |
| Hosuing material | Polycarbonate |
| Heater | optional |
| Dimension (lxwxd) | 400x300x210mm |
| Supply voltage | 230VAC |
| Operating temperature | -40°C ~ +70°C |
| Certifications | IK10 |
| Protection rating | IP66 |
| Manufacturer order number | 226845 |

Additional Productinformation

Related products

| CUS-CSF-060 | with thermostat, for DIN rail mounting Hutschienenmontage |
| --- | --- |
| jetassy-SBT12 | Spleißbox mit Telekom Spleißkassette für 12 Spleiße |
| MDR-100-12 | Netzteil 12VDC / 7,5A für Hutschienenmontage TS-35 |
| MDR-100-48 | Netzteil 48V / 2A für Hutschienenmontage TS-35 |
| MDR-20-12 | Netzteil 12VDC / 1,67A für Hutschienenmontage TS-35 |
| MDR-60-12 | Netzteil 12VDC / 5A für Hutschienenmontage TS-35 |
| MDR-60-48 | Netzteil 48VDC / 1,25A für Hutschienenmontage TS-35 |
| SOL-WST-MB-30 | Pole mounting for CUS-WST and SOL-IQbox series set (2pcs) |
| SOL-WST-SBL | Tightening strap long for SOL-WST-MB, CUS-WST, SOL-IQbox set 2pcs. 1.5m stainless steel |
| SOL-WST-SH/1 | Swivelling lever for profile half-cylinder (1pc.) delivery without lock cylinder |
| SOL-WST-SSK-ES-30 | Erdungsset für Schlösser SOL-WST-SSK für IQbox -budget, -eco, -switch |
| SOL-WST-SSK/1 | Lock with toggle lock (1 piece) with key, for CUS-WST and SOL-IQbox series |
| SOL-WST-WB | Wall mounting for CUS-WST and SOL-IQbox series set (4pcs) wall clearance 10mm |

Recommendations

Services