Specifications text

Article number

226889

Type

MPD-78M2713M0A

Short description

1/2,8" HD Dome Fix, Day&Night 3840x2160, Infrared, WDR, 2,7-13,5mm, 12/24V, IP67

Main Features

1/2.8" 8.64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 3840x2160@15/12,5fps

Motorised varifocal lens 2,7~13,5mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable, Composite SUB

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

DIS (Digital Image Stabilization) function

IP67, IK10, 3-Axis gimbal

For wall or ceiling flush mount

IR Illumination (2 x SMD High Power LED's)

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 119x140mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo Callisto, eneo COAXIZE, eneo KX-Series |
| Resolution Pixels | 1920x1080, 2560x1440, 3840x2160 |
| Resolution Pixels | 1920x1080, 2560x1440, 3840x2160 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,06 Lux at F1,6, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal (motorized) |
| Focal Length | 2,7 - 13,5 mm |
| Iris Control | DC |
| Zoom Factor | 5x |
| Digital Zoom | yes |
| Horizontal Angle of View | 105 - 45° |
| Vertical Angle of View | 59 - 25° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone Cool Gray 1C |
| Color (bubble) | clear |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | no |
| Rotation range | +/-173° |
| Video outputs | AHD, Composite (CVBS), composite (CVBS) SUB, CVI, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog, Digital image stabilization |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 12W |
| Vandal proof | yes |
| Operating temperature | -50°C ~ +50°C |
| Certifications | IK10 |
| Protection rating | IP67 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Signal format | AHD @ 8MP, Composite (FBAS), CVI @ 8MP, EX-SDI @ 8MP, HD-SDI (1.485 Gbps), HD-TVI @ 8MP |
| Camera type | Dome |
| Frame rate | 12,5/15fps @ 3640x2160p, 25/30fps @ 1920x1080p, 25/30fps @ 2560x1440p |
| Aperture range (F) | F1.6 |
| Illumination range | approx. 40m |
| Weight | 1.0kg |
| Manufacturer order number | 226889 |

Additional Productinformation

<h2>Modularer 8MP Multisignal Dome für den Außeneinsatz</h2><p>Der MPD-78M2713M0A liefert extrem hochauflösende Videobilder in 4K Ultra HD. Der eneo 8 MP Dome aus der Callisto-Serie ist für den Einsatz in rauen Umgebungen durch ein qualitativ hochwertiges, robustes Aluminiumgehäuse geschützt. Die Kamera entspricht den Schutzklassen IP67 und IK10 (hoher Schutz vor Staub und Wasser sowie sehr hohe Schlag- und Stoßfestigkeit) und arbeitet bei Temperaturen von -50°C bis 50°C absolut zuverlässig. Ausgestattet mit einem hochwertigen CMOS-Sensor von Sony und einem motorisierten Varifokalobjektiv (Brennweite: 2,7?13,5mm), bietet der 8MP Multisignal Dome Betreibern und Anwendern viel Flexibilität für die Einbindung in bestehende Videosysteme: die Kamera unterstützt die analogen Signalformate HD-TVI, AHD und Composite SUB sowie die digitalen Formate EX-SDI und HD-SDI. Hierfür ist der eneo 8MP Dome mit entsprechenden separaten Ausgängen versehen. <br><br></p><h3>Bildoptimierungsfunktionen für konstant ausgezeichnete Videoqualität</h3><p>Für den 24-Stunden-Einsatz verfügt der 8MP Multisignal Dome über eine integrierte IR-Beleuchtung aus zwei SMD-High-Power-LEDs mit einer Reichweite von ca. 40m. Darüber hinaus ist die Kamera mit verschiedenen Bildoptimierungsfunktionen gegen beleuchtungsbedingte Qualitätseinbußen gewappnet. Der eneo 8MP Dome gleicht mit den Bildoptimierungsfunktionen DOL-WDR und BLC/HLC Gegenlichtkompensation starke Kontraste und Helligkeitsunterschiede aus und erreicht eine gleichmäßig hohe Bildqualität. Mit der mehrdimensionalen digitalen Rauschunterdrückung per 3D-DNR reduziert der 8MP Multisignal Dome Bildrauschen und gleicht die Bildfehler infolge von Farbvermischungen aus. Die Funktion Defog verbessert zudem automatisch die Bildqualität bei Nebel oder Nieselregen, während die digitale Bildstabilisierung (DIS) verwackelte Aufnahmen verhindert, die durch Eigenschwingungen der Kamera hervorgerufen werden können. <br><br></p><h3>Errichterfreundliches Design für die schnelle Installation und Inbetriebnahme</h3><p> Der 3-Achsen-Aufhängering und das modulare Design der Callisto-Serie erlaubt in Verbindung mit der breiten Palette an Installationszubehör eine errichterfreundliche und optimal an die baulichen Gegebenheiten vor Ort angepasste Montage. eneo Zubehörteile wie Anschlussbox (AK-5) Aufbau- und Keiladapter, Wandarm, Sonnendach und eine getönte Kuppel sind ebenfalls bei VIDEOR erhältlich. </p>

Related products

| AK-5 | Camera Junction Box, for Callisto Dome series, IP66 |
| --- | --- |
| DOME-S1 | Sun Shield for eneo Callisto Dome Series |
| DOME-WB1+DOME/ Basis1 | Wall Bracket & Surface mount base, Callisto Dome line |
| DOME-WB3 | Wall bracket for eneo Dome Cameras MED-62xxxxxM0A and IED-62xxxxxM0A |
| DOME/BASIS1 | Surface mount base , Callisto Dome line |
| DOME/WEDGE2 | Wedge Adapter, Callisto Dome Line |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |

Recommendations

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |