Specifications text

Article number

228224

Type

IAM-5SE1004MUD

Short description

Switch, Unmanaged, 120W, 4xPoE 1x RJ45, 1x SFP 100Mbps, Industrial Design, -40°C~+75°C

Main Features

Ethernet PoE Switch, DIN Rail

Uplink ports 1xRJ45, 1xSFP

4x 10/100 PoE Ports,IEEE 802.3af/at

Up to 30W per port

Surge protection IEC61000-4-5 (6KV)

Status LED: Uplink, PoE, Optical port, Power

CCTV mode allows transmission up to 250m @ 10Mbps

Temperature range -40° C ~ +75° C

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 46x110x159mm |
| Type | Unmanaged |
| Series | eneo Switch |
| Mounting | DIN rail, wall mount |
| Blower | no |
| Ethernet interface | 100Base-TX, 10Base-T |
| VLAN | no |
| Ports | 1x 10/100Base-TX, 1x 100 SFP, 4x 10/100Base-TX |
| PoE ports (number) | 4 |
| Management | no |
| Model | DIN rail module |
| Supply voltage | 48~57 VDC |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 120 W |
| Operating temperature | -40°C ~ +75°C |
| Weight | 530g |
| Manufacturer order number | 228224 |

Additional Productinformation

Related products

| SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| --- | --- |

Recommendations

Services