Specifications text

Article number

224194

Type

VM-HD17P

Short description

17" (43cm) LCD Monitor HD, 1280x1024, LED, HDMI, VGA, BNC

Main Features

Professional 17" (43 cm) LCD monitor

Industrial design built to run 24x7x365 continuously

Maximum resolution 1280x1024 pixels

LED Back Light

Fast response time 5ms

Perfect presentation through digital 3D filtering and noise reduction

Different video inputs HDMI, VGA, Composite

Built-in speakers

Supply Voltage: 12VDC

Plastic cabinet including stand

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Refresh rate | 60 Hz |
| Dimensions (HxWxD) | 315,10x382,60x45,30mm |
| Series | eneo Monitor |
| Applications | 24/7 |
| Resolution Pixels | 1280x1024 |
| Resolution Pixels | 1280x1024 |
| Housing color | black |
| Hosuing material | Plastic |
| Backlighting | LED |
| Screen size (inch) | 17" |
| Screen size (cm) | 43.2 cm |
| Aspect ratio | 4:3 |
| Viewing angle vertical | 170° |
| Viewing angle horzontal | 170° |
| Brightness typical | 250 cd/m² |
| Contrast ratio typical | 1000:1 |
| Reponse time typical | 5 ms |
| Video inputs (type) | 2x composite (CVBS), HDMI, VGA |
| Loudspeaker | 2 |
| Video outputs | Composite (CVBS) |
| Audio inputs | 1 |
| Supply voltage | 12VDC |
| Power consumption | 15W |
| Operating temperature | -30°C ~ +85°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 3,6kg |
| Audio output | 1x 3,5mm jack |
| Manufacturer order number | 224194 |

Additional Productinformation

Related products

| VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |

Recommendations

| jetcon-HSA-HSA-1 | Economy HDMI Kabel Stecker A - Stecker A, 1m, HDMI High Speed mit Ethernet |
| --- | --- |
| jetcon-HSA-HSA-10B | HighSpeed HDMI Kabel mit Ethernet, 4K 60Hz, A-A Stecker 10m, schwarz |
| jetcon-HSA-HSA-3 | Economy HDMI Kabel Stecker A - Stecker A, 3m, HDMI High Speed mit Ethernet |

Services