Specifications text

Article number

227050

Type

IPD-68M3611P5A

Short description

1/2,5" Network Dome, Fix, 3840x2160, Day&Night, DOL-WDR, 3,6-11mm, Infrared, Indoor

Main Features

1/2.5" Sony STARVIS CMOS sensor

Resolution max. 3840x2160, 8 Megapixel

Motorised varifocal lens 3.6~11mm

DOL-WDR (120dB) by triple scan images at 30/25fps

VCA video analysis, ONVIF event mapping

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of interest)

Quad Smart Streaming: H.265(+), H.264(+), MJPEG

Slot for Micro SD Card, HTML5 replay

Power supply PoE or 12VDC

Wedge adapter included (10° inclination)

Supports ONVIF Profile S and Profile T

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 94x116mm |
| Series | eneo Callisto Indoor, eneo LN-Series |
| Resolution Standard | 8 Megapixel |
| System | True Day&Night |
| Sensor size | 1/2.5" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 3840x2160 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,2 Lux at F1,5, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal (motorized) |
| Focal Length | 3,6 - 11 mm |
| Iris Control | DC |
| Zoom Factor | 3x |
| Digital Zoom | yes |
| Observe (62 px/m) | 40,4 - 119 m |
| Recognize (125 px/m) | 20,2 - 59,2 m |
| Identify (250 px/m) | 9,8 - 28,8 m |
| Detect (25px/m) | 104 - 295 m |
| Horizontal Angle of View | 84 - 40° |
| Vertical Angle of View | 46 - 21° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Indoor |
| Color (housing) | white |
| Color (bubble) | clear |
| Hosuing material | Plastic |
| Heater | no |
| Blower | yes |
| Rotation range | +/-165° |
| Video outputs | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | no |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSDHC card, microSDXC card |
| Analysis functions | Defog, Field detection, Line detection, Motion detection, Object left,removed |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 8,5W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | yes |
| Operating temperature | -10°C ~ +50°C |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Camera type | Dome |
| Remote configuration | Browser |
| Illumination range | approx. 30m |
| Weight | 300g |
| Manufacturer order number | 227050 |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| IAD-5SG1008MUA | Gigabit Switch, Unmanaged, 120W, 8x PoE, 1xSFP, 1x RJ45, 10/100/1000 Mbps, Desktop |

Recommendations

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |