Specifications text

Article number

227599

Type

VM-FHD12M

Short description

12" (30cm) LCD Monitor FHD, 1920x1080, LED HDMI, VGA, Composite

Main Features

Professional 12" (30 cm) LCD monitor

Industrial design built to run 24x7x365 continuously

Maximum resolution 1920x1080 pixels

LED Back Light

Fast response time 5ms

Perfect presentation through digital 3D filtering and noise reduction

Different video inputs HDMI, VGA, Composite

HD analog multi signal input (TVI, AHD, CVI)

Supply voltage: 12VDC

Metal cabinet including stand

Metallgehäuse inklusive Standfuß

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Refresh rate | 60 Hz |
| Dimensions (HxWxD) | 315x326x43mm |
| Series | eneo Monitor |
| Applications | 24/7 |
| Resolution Pixels | 1920x1080 |
| Resolution Pixels | 1920x1080 |
| Housing color | black |
| Hosuing material | Metal |
| Backlighting | LED |
| Screen size (inch) | 12.5 |
| Screen size (cm) | 30 cm |
| Aspect ratio | 16:9 |
| Viewing angle vertical | 170° |
| Viewing angle horzontal | 170° |
| Brightness typical | 250 cd/m² |
| Contrast ratio typical | 1000:1 |
| Reponse time typical | 5 ms |
| Video inputs (type) | 2x HDMI, AHD, composite (CVBS), CVI, HD-TVI, VGA |
| Loudspeaker | 2 |
| Video outputs | 1x BNC- Signal Loop |
| Audio inputs | 1 |
| Serial interfaces | no |
| Supply voltage | 12VDC |
| Power consumption | 10W |
| Operating temperature | -30°C ~ +85°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 2.3kg |
| Audio output | 1x 3,5mm jack |
| Manufacturer order number | 227599 |

Additional Productinformation

Related products

| VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |

Recommendations

| jetcon-HSA-HSA-1 | Economy HDMI Kabel Stecker A - Stecker A, 1m, HDMI High Speed mit Ethernet |
| --- | --- |
| jetcon-HSA-HSA-10B | HighSpeed HDMI Kabel mit Ethernet, 4K 60Hz, A-A Stecker 10m, schwarz |
| jetcon-HSA-HSA-3 | Economy HDMI Kabel Stecker A - Stecker A, 3m, HDMI High Speed mit Ethernet |

Services