Specifications text

Article number

217918

Type

VM-FHD27MP

Short description

27" (69cm) LCD Monitor, FHD, 1920x1080, LED, HDMI, VGA CVBS, Protection Glass

Main Features

Professional 27" (69 cm) LCD monitor

Industrial design built to run 24x7x365 continuously

Protective glass for harsh environmental conditions

Maximum resolution 1920x1080 pixels

LED Back Light

Fast response time 5ms

Perfect presentation through digital 3D filtering and noise reduction

Different video inputs HDMI,VGA, Composite

Built-in speakers

Supply voltage: 240-100VAC, 50/60Hz

Metal cabinet including stand

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Refresh rate | 60 Hz |
| Dimensions (HxWxD) | 378x617x56mm |
| Series | eneo Monitor |
| Applications | 24/7 |
| Resolution Pixels | 1920x1080 |
| Housing color | black |
| Hosuing material | Metal |
| Backlighting | LED |
| Screen size (inch) | 27" |
| Screen size (cm) | 68,6 cm |
| Aspect ratio | 16:9 |
| Viewing angle vertical | 178° |
| Viewing angle horzontal | 178° |
| Brightness typical | 350 cd/m² |
| Contrast ratio typical | 1000:1 |
| Reponse time typical | 5 ms |
| Video inputs (type) | composite (CVBS), HDMI, VGA |
| Loudspeaker | 2 |
| Audio inputs | 1 |
| Power consumption | 40W |
| Operating temperature | -30°C ~ +85°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 6,7kg |
| Manufacturer order number | 217918 |

Additional Productinformation

Related products

| VM-LCDWMB1 | Universal Wall Mount for LCD Monitors, 32" to 65", VESA support up to 35Kg max. |
| --- | --- |
| VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |

Recommendations

| jetcon-HSA-HSA-1 | Economy HDMI Kabel Stecker A - Stecker A, 1m, HDMI High Speed mit Ethernet |
| --- | --- |
| jetcon-HSA-HSA-10B | HighSpeed HDMI Kabel mit Ethernet, 4K 60Hz, A-A Stecker 10m, schwarz |
| jetcon-HSA-HSA-3 | Economy HDMI Kabel Stecker A - Stecker A, 3m, HDMI High Speed mit Ethernet |

Services