Specifications text

Article number

230036

Type

IAM-5SE1016IMA

Short description

Gigabit Switch, Managed, 360W Layer 3, 16xPoE, 4x SFP 10G, Industrial Design

Main Features

16-Port Gigabit Switch Managed L3, Fanless

4x SFP 10 Gigabit Uplink Ports

PoE Standard IEEE 802.3af/at

Up to 30W per port, total ?360W

Power supply not included

Fast Ring-Redundant, Reconfiguration <20ms

ESD protection IEC61000-4-5 standard

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password protection

Relay Alarm for power, cable link and PoE link down

Temperature range -40° C ~ +75° C

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 95,6x163x110mm |
| Type | Layer3 Switch |
| Series | eneo Switch |
| Mounting | DIN rail, wall mount |
| Blower | no |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | yes |
| Ports | 16x 10/100/1000Base-T, 4x 10GBit SFP, Console RJ45-RS232, OOB 10/100Base-TX |
| PoE ports (number) | 16 |
| Management | yes |
| Model | DIN rail module |
| Supply voltage | 48~57 VDC |
| Power consumption | 380W max. |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 360 W |
| Operating temperature | -40°C ~ +75°C |
| Weight | 1.7kg |
| Manufacturer order number | 230036 |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

| jetpower-NDR-240-48 | Netzgerät für DIN Rail Montage 230VAC Eingang, 48VDC Ausgang max. 240W |
| --- | --- |

Services