Specifications text

Article number

229215

Type

VM-SD8M

Short description

8" (20,3cm) LCD Monitor SD, 1024x768, LED, HDMI, VGA, Composite

Main Features

Professional 8" (23,3 cm) LCD monitor

Industrial design built to run 24x7x365 continuously

Maximum resolution 1024x768 pixels

LED Back Light

Fast response time 5ms

Perfect presentation through digital 3D filtering and noise reduction

Different video inputs HDMI, VGA, Composite

Supply voltage: 12VDC

VESA Standard 75x75mm

Metal cabinet including stand

Metallgehäuse inklusive Standfuß

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Refresh rate | 60 Hz |
| Dimensions (HxWxD) | 148,8x189,9x43mm |
| Series | eneo Monitor |
| Applications | 24/7, Industrie |
| Resolution Pixels | 1024x768 |
| Resolution Pixels | 1024x768 |
| Housing color | black |
| Hosuing material | Metal |
| Backlighting | LED |
| Screen size (inch) | 8 |
| Screen size (cm) | 20.3 cm |
| Aspect ratio | 4:3 |
| Viewing angle vertical | 150° |
| Viewing angle horzontal | 160° |
| Brightness typical | 250 cd/m² |
| Contrast ratio typical | 700:1 |
| Reponse time typical | 5 ms |
| Video inputs (type) | 2x composite (CVBS), HDMI, VGA |
| Loudspeaker | 2 |
| Video outputs | Composite (CVBS) |
| Audio inputs | 1 |
| Serial interfaces | no |
| Supply voltage | 12VDC |
| Power consumption | 10W |
| Operating temperature | -30°C ~ +85°C, @ Humidity 10%-90% (non-condensing) |
| Weight | 2.5 kg |
| Audio output | 1x 3,5mm jack |
| Manufacturer order number | 229215 |

Additional Productinformation

Related products

| VMC-LCD/WCMB-1 | Wall/Ceiling Bracket for LCD Monitors 15"/17"/19" P/CP/CM/HPC/HMC1 Series |
| --- | --- |

Recommendations

| jetcon-HSA-HSA-1 | Economy HDMI Kabel Stecker A - Stecker A, 1m, HDMI High Speed mit Ethernet |
| --- | --- |
| jetcon-HSA-HSA-10B | HighSpeed HDMI Kabel mit Ethernet, 4K 60Hz, A-A Stecker 10m, schwarz |
| jetcon-HSA-HSA-3 | Economy HDMI Kabel Stecker A - Stecker A, 3m, HDMI High Speed mit Ethernet |

Services