Specifications text

Article number

232562

Type

ISM-62F0028W0A

Short description

1/2,8" Network Board Camera 1920x1080, Day/Night, WDR, 2,8mm, H.265, PoE, 12VDC

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 1920x1080 (Full HD)

Lens fixed focal length 2.8mm

WDR (120dB), BLC, 3DNR

Tripple Streaming (H.265, H.264, MJPEG)

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Object classification (person, car, bike)

Supports ONVIF Profile S

Operating temperature -20°C +50°C

Dimensions 50x50x59mm

Power supply PoE or 12VDC

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | see drawing |
| Camera Type | Board Camera |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, H.265 |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,002 Lux at F1,6 |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 2,8 mm |
| Iris Control | DC |
| Observe (62 px/m) | 17,7 m |
| Recognize (125 px/m) | 8,6 m |
| Identify (250 px/m) | 4,1 m |
| Detect (25px/m) | 44,9 m |
| Horizontal Angle of View | 113° |
| Vertical Angle of View | 61° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | no |
| Housing | without housing |
| Blower | no |
| Video outputs | no |
| Alarm inputs | 1 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | no |
| Analysis functions | Human&Vehicle Detection (PD&VD), Motion detection |
| Mounting | Einbaumodul |
| Supply voltage | 12VDC, PoE |
| Power consumption | 3.6W |
| Power over Ethernet | IEEE 802.3af |
| Operating temperature | -20°C ~ +50°C |
| ONVIF | ONVIF Profile S |
| Low Speed Shutter | no |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | Platine |
| Audio | 2 Way Audio |
| Remote configuration | Browser, Remote software |
| Weight | 260g |
| Manufacturer order number | 232562 |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |