Specifications text

Article number

233036

Type

VM-HD19MPA

Short description

19" (48cm) LCD, Industrial Monitor, 1280x1024, HDMI, VGA, Composite, Protection Glass

Main Features

Professional 19" (48 cm) LCD monitor

Built to run 24x7x365 continuously (Industrial Level)

Including protection glass, Anti-Glare-Display 3H

Maximum resolution 1280x1024 pixels

LED Back Light

Fast response time 5ms

Different video inputs HDMI, VGA, BNC (CVBS)

Audio In- and Output

Built-in speakers

Metal cabinet including wall mount

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Refresh rate | 60 Hz |
| Dimensions (HxWxD) | 340,6x401,28x49mm |
| Series | eneo Monitor |
| Applications | 24/7 |
| Resolution Pixels | 1280x1024 |
| Resolution Pixels | 1280x1024 |
| Housing color | black |
| Hosuing material | Metal |
| Backlighting | LED |
| Screen size (inch) | 19" |
| Screen size (cm) | 48 cm |
| Aspect ratio | 5:4 |
| Viewing angle vertical | 178° |
| Viewing angle horzontal | 178° |
| Brightness typical | 250 cd/m² |
| Contrast ratio typical | 1000:1 |
| Reponse time typical | 14 ms |
| Video inputs (type) | composite (CVBS), HDMI, VGA |
| Loudspeaker | 2 |
| Video outputs | Composite (CVBS) |
| Audio inputs | 1 |
| Serial interfaces | no |
| Power consumption | 25W |
| Operating temperature | 10°C ~ +40°C |
| Weight | 4,1kg |
| Manufacturer order number | 233036 |

Additional Productinformation

Related products

| VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |

Recommendations

| jetcon-HSA-HSA-1 | Economy HDMI Kabel Stecker A - Stecker A, 1m, HDMI High Speed mit Ethernet |
| --- | --- |
| jetcon-HSA-HSA-10B | HighSpeed HDMI Kabel mit Ethernet, 4K 60Hz, A-A Stecker 10m, schwarz |
| jetcon-HSA-HSA-3 | Economy HDMI Kabel Stecker A - Stecker A, 3m, HDMI High Speed mit Ethernet |

Services