Specifications text

Article number

233904

Type

INR-18N040005A

Short description

Netzwerk Video Rekorder, 4x IP PoE, H.265, HDMI, VGA, 1xSATA, ohne HDD

Main Features

Maximum resolution 8MP (recording)

Recording and research AI analysis functions

Recording modes continuous, manual or AI-based

Maximum data rate input 50Mbps, output 64Mbps

Various options for alarm messages

4 x RJ45 PoE port (45 Watt PoE budget total)

Maximum 1 internal hard disk possible

Supports 4 x alarm inputs and 1 x alarm output

2 x USB inputs, 1 x RS-485, 1 x audio input and output

eSATA port for external storage expansion

NetDrive or cloud storage (Google Drive/Dropbox)

Remote access via browser, eneo Insight or Insight mobile

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 300X227X53mm |
| Series | eneo IN-Series |
| Blower | no |
| Channels total | 4 |
| IP channels | 4 |
| Max. recording resolution | 3840x2160 |
| Compression | H.264, H.264+, H.265, H.265+ |
| Spot monitor | no |
| Monitor Outputs | HDMI (4K), VGA |
| Network | 1x 10/100/1000 Base-T RJ45 |
| Alarm inputs | 4 |
| Alarm output | 1 |
| Audio support | yes |
| Audio inputs | 1 |
| Serial interfaces | RS-485 |
| Max. internal HDD capacity | 10 TB |
| Internal HDD capacity | 0 TB |
| Integrated writer | no |
| Interfaces external storage | USB |
| HDD slots | 1x SATA |
| HDD hotswap | no |
| USB ports | 2x USB 2.0 |
| RAID | n/a |
| Data tranfer rate max. | 114 Mbps |
| Analysis functions | Depending on connected IP camera |
| Remote access | Android, iPhone, remote software, WEB Browser (Java) |
| 19" rack mount | no |
| Supply voltage | 48VDC |
| Power consumption | 5W, without HDD |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 45 W |
| Operating temperature | 0°C ~ +40°C |
| ONVIF | ONVIF Profile S, yes |
| Data transfer rate outgoing | 64 Mbps |
| Remote configuration | Remote software |
| Weight | 1,03kg |
| Playback | 4 channel split, full screen, sequence |
| Network connection | 1 x 10/100/1000BASE-T, 4x PoE RJ45 10/100 Base-T |
| Data transfer rate incoming | 50 Mbps |
| Audio output | 1 |
| Operating system | Linux |
| Manufacturer order number | 233904 |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series IN-Serie |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |
| INR-eSATA | HDD Box, eSATA, für max 4 HDD max. 8TB pro HDD, ohne HDD |

Recommendations

| DS223 | Network Attached Storage, 2-Bay, Hotswap, ohne HDD, Quad core, 2GB RAM |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |

Services

| CON-HDD-eneo | Hard disk installation service for eneo |
| --- | --- |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |
| CON-NVR-KON-04 | Preconfiguration 1x Network Video Recorder with up to 4 IP cameras |