Specifications text

Article number

233903

Type

MNR-18N04000A

Short description

Hybrid HD Video Recorder 4-channel analogue, 2-ch. IP, HDMI, 1xSATA, w/o HDD

Main Features

4-channel pentaplex multisignal recorder

Supports IP, HD-TVI, AHD, CVBS signals

Max. 6 channels (4x analog + 2x IP)

Recording speed 28fps@8MP,48fps@5MP,120fps@2MP

Monitor outputs HDMI (4K),VGA and spot monitor CVBS

Maximum 1 internal hard disk possible

PTZ control via coaxial (UTC)

Audio/alarm inputs and outputs

USB 2.0 and RS-485 interface

Compression methods: H.264, H.265

Remote access via app, browser and CMS

USB mouse included

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4260229840252 |
| Dimensions (HxWxD) | 300X227X53mm |
| Serial interfaces protocols | Pelco D¦Pelco P¦coax1¦coax2 |
| Series | eneo IN-Series |
| Housing | Metal/Plastic |
| Color | black |
| Blower | no |
| Video inputs (amount) | 4 |
| Channels total | 6 |
| IP channels | 2 (6 IP only) |
| Max. recording resolution | 3840x2160 |
| Recording rate max. | @1920x1080 |
| Compression | H.264, H.265 |
| Main video outputs | HDMI (4K), VGA |
| Spot monitor | composite (CVBS) |
| Network | 10/100 Base-Tx, RJ45 |
| Alarm inputs | 4 |
| Alarm outputs | 1 |
| Audio inputs | 4 |
| Serial interfaces | Coaxitron (HD Protocol), RS-485 |
| Max. internal HDD capacity | 10 TB |
| Internal HDD capacity | 0 TB |
| Integrated writer | no |
| Interfaces external storage | USB |
| HDD slots | 1x SATA |
| HDD hotswap | no |
| RAID | n/a |
| Data tranfer rate max. | 48 Mbps |
| Analysis functions | Additional functions depending on connected IP camera, Human&Vehicle Detection (PD&VD), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), The recorder hardware (on board) provides the following analysis functions for analog cameras |
| Remote access | Microsoft Internet Explorer, remote software |
| 19" rack mount | no |
| Supply voltage | 12VDC |
| Power consumption | 4.7W, without HDD |
| Operating temperature | 0°C ~ +40°C |
| ONVIF | ONVIF Profile S, yes |
| Data transfer rate outgoing | 36 Mbps |
| Operation mode | Pentaplex |
| Signal format | AHD @ 8MP, Composite (FBAS), HD-TVI @ 8MP, IP @ 8MP |
| Weight | 1,03kg |
| Playback | 4 channel split, full screen |
| Data transfer rate incoming | 12 Mbps |
| Audio output | RCA |
| Operating system | Linux |
| Manufacturer order number | 233903 |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series IN-Serie |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |

Services

| CON-eneo-KON-04 | Preconfiguration 1x eneo recorder with up to 4 eneo IP cameras |
| --- | --- |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |