Specifications text

Article number

233697

Type

MEB-55M2812M0A

Short description

1/2.8" HD Camera, Day&Night, 2592x1944, Infrared 2,7-13,5mm, 12VDC, IP67

Main Features

1/2.8" CMOS Image Sensor

Resolution max. 2592x1944

multi-signal: HD-TVI, AHD, CVI, CVBS

2,7-13,5 mm lens with variable focal length (motorized)

Day/night switching (ICR)

2D+3D DNR (Digital Noise Reduction)

Automatic exposure control (1/30s~1/50,000s)

Lens shading correction

IR LED illumination integrated

For outdoor use degree of protection IP67

12VDC power supply, NDAA compliant

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4,26023E+12 |
| Dimensions (HxWxD) | 83,88x87,4x242mm |
| Series | eneo COAXIZE, eneo IN-Series |
| Resolution Pixels | 1920x1080, 2560x1440, 2592x1944, 960x540 |
| Resolution Pixels | 1920x1080, 2560x1440, 2592x1944, 960x540 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,05 Lux at F1,2 |
| Digital Noise Reduction | 2D-DNR, 3D-DNR |
| Lens Type | Varifocal (motorized) |
| Focal Length | 2,7 - 13,5 mm |
| Iris Control | DC |
| Zoom Factor | 4x |
| Digital Zoom | no |
| Observe (62 px/m) | 24,3 - 104,5 m |
| Recognize (125 px/m) | 11,5 - 52,2 m |
| Identify (250 px/m) | 5,2 - 25,6 m |
| Detect (25px/m) | 62,5 - 260,3 m |
| Horizontal Angle of View | 110,6 - 32,6° |
| Vertical Angle of View | 57,3 - 18,3° |
| MOD (Mininum Object Distance) | 0,8 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone P 179-1, RAL9003 |
| Hosuing material | Aluminium |
| Heater | no |
| Video outputs | AHD, Composite (CVBS), HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 6.5W |
| Vandal proof | no |
| Operating temperature | -30°C ~ +55°C |
| Protection rating | IP67 |
| Low Speed Shutter | no |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | no |
| Signal format | AHD @ 2MP, AHD @ 4MP, AHD @ 5MP, Composite (FBAS), CVI @ 2MP, CVI @ 4MP, HD-TVI @ 2MP, HD-TVI @ 4MP, HD-TVI @ 5MP |
| Camera type | Bullet |
| Frame rate | 20fps @ 2592x1944p, 25/30fps @ 1920x1080p, 25/30fps @ 2560x1440p |
| Aperture range (F) | F1.8~F3.3 |
| Illumination range | approx. 35m |
| Weight | 745g |
| Manufacturer order number | 233697 |

Additional Productinformation

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for Bullet or Fix Dome IN-Series cameras |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |
| WB-24 | Wall bracket for Dual camera connection box AK-24 |

Recommendations

Services