Specifications text

Article number

237369

Type

IAD-5SG1004MUC

Short description

Gigabit Switch, Unmanaged, 60W, 4x PoE, 1x RJ45, 1x SFP, 10/100/1000 Mbps, Desktop

Main Features

Gigabit PoE switch, desktop application

4x 10/100/1000 PoE+ ports, IEEE 802.3af/at

Gigabit uplink ports: 1x RJ45, 1x SFP

Up to 30W per port, total ?60W.

Overvoltage protection IEC61000-4-5 (6KV).

Temperature range (operation) -5°C~45°C

CCTV mode enables transmission up to 250m @10Mbps

CCTV mode supports PoE watchdog function

Plug and Play, no settings required

Metal housing

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 27x135x86mm |
| Type | Unmanaged |
| Mounting | desktop |
| Blower | no |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | no |
| Ports | 1x 10/100/1000Base-T, 1x 1000Base-X SFP, 4x 10/100/1000Base-T (PoE) |
| PoE ports (number) | 4 |
| Management | no |
| Model | Desktop |
| Supply voltage | 48~57 VDC |
| Power consumption | 65W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 60 W |
| Operating temperature | -5°C ~ +45°C |
| Weight | 300g |
| Manufacturer order number | 237369 |

Additional Productinformation

Related products

| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| --- | --- |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

| jetpower-NDR-240-48 | Netzgerät für DIN Rail Montage 230VAC Eingang, 48VDC Ausgang max. 240W |
| --- | --- |

Services