Specifications text

Article number

235524

Type

ISM-52F0021WMA

Short description

1/2,8" Network Module Camera, 1920x1080, Day&Night,, WDR, 2,1mm, H.265, PoE, 12VDC

Main Features

1/2,8" 2.0 MP CMOS-Sensor

Resolution max. 1920x1080 (Full HD)

Board Camera 50x50x50mm

Quad Streaming (H.265, H.264, MJPEG)

True Day & Night (TDN)

Noise Reduction (2D-/3D-DNR)

WDR 120dB, Smart BLC

Slot for Micro SD memory card

ONVIF conformance, PoE IEEE802.af, Cross Web-Browsing

Audio-, Alarm- In- and Outputs

Privacy zones, password protection, Cross Web-Browsing

NDAA compliant

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 50x50x50mm |
| Camera Type | Board Camera |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, H.265, MJPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 2,1 mm |
| Iris Control | no |
| Digital Zoom | yes |
| Horizontal Angle of View | 140° |
| Vertical Angle of View | 78° |
| MOD (Mininum Object Distance) | 0,3 m |
| llumination | no |
| Housing | without housing |
| Blower | no |
| Video outputs | no |
| Alarm inputs | 1 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Motion detection, Perimeter Intrusion Detection (PID), Tampering Detection (TD) |
| Mounting | Einbaumodul |
| Supply voltage | 12VDC, PoE |
| Power consumption | 4.3W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | 0°C ~ +40°C |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | True WDR |
| Motion Detector | yes |
| Camera type | Platine |
| Audio | 2 Way Audio |
| Remote configuration | Browser |
| Weight | 125g |
| Manufacturer order number | 235524 |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |