Specifications text

Article number

238647

Type

IND-58F0028MTLA

Short description

Network Dome Fix, Turret, Low Light, 3840x2160, Day/Night WDR, AI, 2,8mm, 12VDC, PoE, IP67

Main Features

1/1.8" Sony STARVIS CMOS sensor

Very good detail recognition even in dark areas

Maximum resolution 3840x2160 pixels, 30fps

AI analysis functions including object classification

WDR 120dB, 3D DNR noise reduction

Lens fixed focal length 2.8mm

Smart codec H.265/H.264, multi-streaming

Browser-independent (Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G

Local storage via micro SD card

NDAA compliance

White light LED illumination, IP67

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 97,9x115,5mm |
| Series | eneo IN-Series |
| Camera Type | Face recognition, License plate recognition, People Count, Retail Intelligence |
| Resolution Standard | 8 Megapixel |
| System | 24h color |
| Sensor size | 1/1.8" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 3640x2160 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | no |
| Sensitivity | 0.0005 Lux @ F1,0 |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 2,8 mm |
| Iris Control | DC |
| Digital Zoom | yes |
| Horizontal Angle of View | 112° |
| Vertical Angle of View | 61° |
| MOD (Mininum Object Distance) | 0,7 m |
| llumination | White light |
| Housing | Outdoor |
| Color (housing) | Pantone P 179-1, RAL9003 |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | no |
| Video outputs | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Attribute Detection (AD), Cross Counting (CC), Crowd Density Detection (CD), Face Attendance (FA), Face Detection (FD), Face Recogntion (FR), Heat Map (HM), Human&Vehicle Detection (PD&VD), License Plate Detection (LPD), License Plate Recognition (LPR), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Queue Lenght Detection (QD), Rare Sound Detection (RSD), Stationary Object Detection (SOD) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 12VDC: max. 5,1W , PoE: max. 6,5W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -30°C ~ +55°C, @ Humidity 95% or less (non-condensing) |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP67 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | Dome |
| Audio | 2 Way Audio, built-in loudspeaker, built-in microphone |
| Remote configuration | Browser |
| Illumination range | approx. 25m |
| Weight | 575g |
| Manufacturer order number | 238647 |

Additional Productinformation

<p>The <strong>eneo 8 megapixel dome camera</strong> offers advanced surveillance technology, ideal for use in demanding environments. Equipped with a <strong>highly sensitive</strong> <strong>1/1.8" Sony STARVIS CMOS sensor</strong>, it guarantees exceptional detail detection, even in dimly lit areas. Thanks to the 4K resolution of 3840x2160 pixels at 30 frames per second, the camera delivers razor-sharp and smooth images that capture every movement precisely.</p><p>The <strong>integrated AI analysis functions</strong>, including object classification, face and license plate recognition and people counting, enable smart and automated surveillance. These functions are ideal for use in retail, traffic monitoring and security applications. Thanks to the <strong>WDR of 120 dB</strong> and the <strong>3D-DNR noise reduction</strong>, excellent images are achieved even in challenging lighting conditions and backlight.</p><p>The fixed focal length of 2.8 mm offers comprehensive coverage with a horizontal angle of view of 112 °. With the <strong>integrated white light LED lighting</strong> and a range of up to 25 meters, surveillance is ensured even in complete darkness.</p><p>Another highlight is the <strong>versatility in connectivity</strong>: Thanks to <strong>ONVIF profiles S, T and G</strong>, the camera can be easily integrated into existing surveillance systems. It is browser-independent, supports all common web browsers and offers <strong>2-way audio</strong> for direct communication. Local storage via a MicroSD card provides additional flexibility when backing up data.</p><p>With its <strong>robust IP67 aluminum housing</strong>, the camera is ideal for outdoor use and can withstand extreme weather conditions. Its compact design and low weight of 575 g enable easy installation on walls or ceilings.</p><p>The <strong>eneo 8 megapixel dome camera</strong> combines advanced technology with reliable performance - perfect for professional use in any environment.</p>

Related products

| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| DOME-WB7 | Wall bracket for Mini Dome IND-42M2808M0A |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| jetcon-Cat-Cat-FC | Kabelverbinder AWG27-22, IP67 Feldkonfektionierbar |
| jetcon-RJ45S-Cat6-FC | RJ45 Stecker Cat.6 AWG22-27, Feldkonfektionierbar, schwarz |
| jetcon-RJ45S-Cat6-small-FC | RJ45 Stecker Cat.6 AWG22-26, Feldkonfektionierbar, schmale und kurze Ausführung |
| jetcon-RJ45S-Cat6A-FC | RJ45 Stecker Cat.6A AWG22-27 Feldkonfektionierbar, blau |
| jetpower-PSU-XS-11C | Universal Netzteil 12V/1,5A ID 2.1 x OD 5.5, 11mm |
| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |