Specifications text

Article number

236317

Type

INT-8SF0003M0A

Short description

Network Hybrid Bullet Camera Day/Night, 2592x1944 optical, 256x192 thermal, IP66, PoE, 12V

Main Features

Dual network bullet camera

Optical resolution 2592x1944 pixels

Thermal resolution 256x192 pixels

Fixed focal 3.6mm (optical) H:81°, V:57°

Fixed focal 3.2mm (thermal) H:56°, V:42°

AI analysis functions including object classification (optical)

Temperature alarm measuring range -20°~ +150° (±8°)

Supports audio and alarm inputs and outputs

Local storage via micro SD card

Supports ONVIF profiles S, G and profile T

NDAA compliance

Suitable for outdoor use IP66

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 108x91x314,6mm |
| Series | eneo IN-Series |
| Camera Type | Thermal Hybrid Bullet |
| Resolution Standard | 5 Megapixel |
| Resolution Pixels | 2560x1944, 256×192 Thermal Modul |
| Resolution Pixels | 2560x1944, 256×192 Thermal Modul |
| System | thermal image, True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Video compression | H.264, H.265, M-JPEG |
| Image resolution max. | 2592x1944 |
| Image Transmission Rate | 25 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,003 Lux @ F1,6 AGC on |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 3,2 mm, 3,6 mm |
| Iris Control | DC |
| Horizontal Angle of View | 56°, 81° |
| Vertical Angle of View | 42°, 57° |
| MOD (Mininum Object Distance) | 0,3 m, 2 m |
| llumination | Infrared, White light |
| Housing | Outdoor |
| Color (housing) | Pantone P 179-1, RAL9003 |
| Hosuing material | Aluminium |
| Heater | no |
| Video outputs | no |
| Alarm inputs | ja |
| Alarm outputs | yes |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Cross Counting (CC), Crowd Density Detection (CD), Face Detection (FD), Fire detection, Heat Map (HM), Human (PD), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Queue Lenght Detection (QD), Rare Sound Detection (RSD), Stationary Object Detection (SOD), Temperature exception alarm, Vehicle (VD) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 10,2W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -30°C ~ +60°C |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP67 |
| Thermal resolution | 256x192 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Camera type | Bullet |
| Audio | built-in loudspeaker |
| Remote configuration | Browser |
| Illumination range | approx. 15m, approx. 30m, Infrared, White Light |
| Weight | 1,45kg |
| Manufacturer order number | 236317 |

Additional Productinformation

<p>Early fire detection can be used in a variety of areas to respond to fire threats early, minimizing property damage, personal injury, and business interruption.</p><p>Here are some possible areas of application:</p><p><strong>Residential buildings:</strong> To protect residents and property from fires that could be caused by household appliances, candles, smoking, or electrical problems.</p><p><strong>Office buildings:</strong> To ensure employee safety and prevent data loss and property damage from fire.</p><p><strong>Industrial facilities:</strong> Particularly important in manufacturing facilities, warehouses, and factories where flammable materials and machinery pose a higher fire risk.</p><p><strong>Critical infrastructure:</strong> Hospitals, schools, airports, train stations, and other important facilities where a fire could have catastrophic consequences.</p><p><strong>Agricultural operations:</strong> To protect crops, livestock, and farm buildings from fire.</p><p><strong>Server rooms and data centres:</strong> To ensure the protection of sensitive data and expensive hardware that could be destroyed by fire.</p><p><strong>Vehicles and ships:</strong> In transportation such as buses, trains, airplanes, and ships to ensure the safety of passengers and cargo.</p><p><strong>Historic buildings and museums:</strong> To protect priceless cultural assets and historical artifacts from fire.</p><p><strong>Large events and public places:</strong> To protect crowds at concerts, sporting events and other public gatherings.</p><p><strong>Forests and nature reserves:</strong> For early detection of forest fires to minimize environmental and property damage and prevent the spread of fires.</p><p>In addition, this camera supports various AI functions for the thermal and optical camera modules. Please refer to our AI Matrix in the download area for further details.</p>

<p>DRI distance human: detection 126 m, recognition 32 m, identification 21 m</p><p>DRI distance vehicle: detection 421 m, recognition 90 m, identification 53 m</p><p>DRI stands for Detection, Recognition and Identification and is a generally accepted set of standards that attempts to measure a distance at which a thermal sensor can produce an image of a particular target. These standards were originally developed by the US Army in the 1950s. The model is based on a 50 percent probability of hitting targets under ideal conditions.</p>

Related products

| AK-20 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| --- | --- |
| AK-21 | Camera connection box for Wall and ceiling mounting, IN Series Bullet Cameras |
| AK-24 | Dual camera connection box for Bullet or Fix Dome IN-Series cameras |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |
| WB-24 | Wall bracket for Dual camera connection box AK-24 |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| jetpower-PSU-XS-11C | Universal Netzteil 12V/1,5A ID 2.1 x OD 5.5, 11mm |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |