Specifications text

Article number

240909

Type

ExCam IPQ6075 T08-TNXCD-N.H-005.N-P

Short description

Network PTZ dome camera in stainless steel housing accordance with RL 2014/34/EU (ATEX) and IECEx, 1080p, 40x optical zoom, 0°C/60°C

Main Features

1/2.8" CMOS sensor with 2 MP (1920 x 1080)

Powerful motorized zoom autofocus lens (40x optical)

AISI 316L stainless steel

Certified for hazardous areas according to ATEX, IECEx, INMETRO, EAC-Ex

Scratch and heat-resistant LEXAN dome for perfect optical quality

Lightfinder and WDR

Automatic temperature control

Fast pan and tilt drive (0.05 - 450°/s)

Protection class IP66/68 (IEC 60529)

Made in Germany

Other Features (Specifications)

| Brand | SAMCON |
| --- | --- |
| Series | ExCam |
| Camera Type | Explosionsgeschützt |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, H.265, M-JPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,1 Lux bei 30 URE F1.6 |
| Digital Noise Reduction | yes |
| Lens Type | auto focus zoom |
| Focal Length | 4,25 - 170 mm |
| Digital Zoom | 12x |
| Horizontal Angle of View | 65 - 2° |
| Housing | Explosionsgeschützt |
| Color (housing) | Edelstahl |
| Hosuing material | Stainless steel |
| Heater | no |
| Video outputs | no |
| Alarm inputs | nein |
| Audio support | no |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Supply voltage | PoE |
| Power over Ethernet | IEEE 802.3at |
| Operating temperature | 0°C ~ +60°C |
| ONVIF | ONVIF Profile S |
| Certifications | ATEX, EACex, IECEx, INMETRO, MASC, UKEX |
| Protection rating | IP66/68 |
| Low Speed Shutter | yes |
| Backlight Compensation | n/a |
| Wide Dynamic Range (WDR) | 120dB |
| Motion Detector | yes |
| Manufacturer order number | 23040117 |

Additional Productinformation

<p>Bei der<strong> ExCam IPQ6075</strong> handelt es sich um eine leistungsfähige IP-Dome-Kamera der allerneuesten Generation mit 2-Megapixel-Auflösung bei <strong>1920 x 1080p</strong> Punkten. Die Hochgeschwindigkeits-PTZ-Kamera kann sich endlos um die eigene Achse drehen; in der Neige deckt sie einen Bereich von 166° ab, wobei das Bild automatisch gewendet wird. Hervorzuheben ist die hohe Geschwindigkeit und Präzision, mit der dies geschieht. Dank der Lightfinder-Technologie erhält man gestochen scharfe Bilder trotz schwieriger Lichtverhältnisse. Aber nicht nur die hervorragende Lichtempfindlichkeit macht die ExCam IPQ6075 so besonders, auch der <strong>40-fach optische Zoom</strong> sorgt für Detailschärfe und brillante Weitwinkelaufnahmen.</p><h2>Explosionsgeschützte Dome-Kamera mit ATEX-, IECEx- und EAC-Ex-Zulassung</h2><p class="align-justify">Die ExCam-Reihe ist sowohl im Rahmen der europäischen (<strong>ATEX</strong>) als auch der internationalen Richtlinie (<strong>IECEx</strong>) zertifiziert. Das ex-geschützte Gehäuse ist für die ATEX Gruppe II für die Zonen 1, 2 sowie 21 und 22 einschließlich der Explosionsgruppen IIC / IIIC zugelassen. Des Weiteren verfügt sie nun auch über die <strong>EAC-Ex</strong> Zulassung.</p><p class="align-justify">Bei der Entwicklung der ExCam IPQ6075 wurde sehr hoher Wert auf <strong>Sicherheit </strong>sowie <strong>mechanische Präzision</strong> und hochwertigen Edelstahl gelegt. Zudem stand ein modularer Aufbau im Vordergrund der Entwicklung.</p><div id="c11507" class=""><div class="ce-textpic ce-left ce-intext"><div class="ce-bodytext"><h2>Temperaturen</h2><p class="align-justify">Auch im Hinblick auf die zulässigen Umgebungstemperaturen setzt die ExCam IPQ6075 Maßstäbe: Die Grenzwerte liegen bei -50°C und +60°C.<br />Um bei -50°C frostfrei zu bleiben, setzen wir PTC-Keramiken zum Heizen ein.</p><h2>Minimaler Installationsaufwand & Power over Ethernet (PoE)</h2><p class="align-justify">Eine Besonderheit der ExCam IPQ6075 liegt darin, dass sowohl Daten als auch die Spannungsversorgung in nur einem einzigen Kabel geführt werden können. Somit wird für den Anschluss im sicheren Bereich nur noch ein PoE-Switch oder ein PoE-Midspan benötigt. Die Stromversorgung der Kameras erfolgt per Power over Ethernet (PoE gemäß IEEE 802.3at) über das Netzwerk, die kostspielige Installation einer separaten Energie-Versorgungsleitung entfällt.</p></div></div></div><div id="c11485" class=""><h2>Ex-Kamera mit HD-Auflösung in Dome-Kameragehäuse</h2><div class="ce-textpic ce-left ce-intext"><div class="ce-bodytext"><p class="align-justify">Mit ihrer Länge von 378 mm und einem Durchmesser von 195 mm ist die ExCam IPQ6075 kleiner als manche Kameramodelle für sichere Bereiche - bei leistungsstärkeren Funktionen. Dank progressiver Abtastung ist die ExCam IPQ6075 in der Lage, hochauflösende Bilder bewegter Objekte mit bis zu 60 Frames pro Sekunde zu liefern: Ohne Bewegungsunschärfe! Die ExCam IPQ6075 liefert gestochen scharfe Aufnahmen in HD Auflösung und exzellente Weitwinkelaufnahmen.</p></div></div></div><iframe width="560" height="315" src="https://www.youtube.com/embed/yiH4IoG4VsU?si=3xR45_boluhAlfyh&start=76" title="YouTube video player" frameborder="0" allow="accelerometer; autoplay; clipboard-write; encrypted-media; gyroscope; picture-in-picture; web-share" referrerpolicy="strict-origin-when-cross-origin" allowfullscreen></iframe>

Related products

| Deckenmontageadapter CMB-TNXCD | Adapter for devices from the T08-VA4 and TNXCD series, also suitable for horizontal wall mounting or Pole mounting of the ExCam IPP3827 |
| --- | --- |
| ExConnection Rail T04.5-D.BOR-4PoE++-1000Base | High-performance PoE switch for the Ex area pressure-resistant encapsulated housing with sight glass, Explosion protection approval 2014/34/EU |
| ExConnection Rail T04.6-D.BOR-8PoE++-1000Base | High-performance PoE switch for the Ex area pressure-resistant encapsulated housing with sight glass, Explosion protection approval 2014/34/EU |
| ExTB-3 Klemmkasten für ExCam Serie | Model: T12-M20-M20-M20 |
| Mastadapter PMB | Mast adapter for TNXCD & T08-VA1.x/2.x wall bracke suitable for pole diameters between 100 and 150mm |
| SKD02-T Kabel Kit 30 m | 30 meter SKD02-T system cable Digital special cable with Ex approval according to DIN EN 60079-14 |
| SKDP03-T Kabel Kit 30 m | 30 meter SKDP03-T system cable Digital special cable with Ex approval according to DIN EN 60079-14 |
| Wandausleger WMB-TNXCD | Wall bracket for T08-TNXCD series devices Suitable for suspended mounting |

Recommendations

Services