Specifications text

Article number

75802

Type

VZT-21B/TB

Short description

Passive Twisted Pair Transmitter or Receiver, Compos. In/Output, BNC-T.Block

Main Features

Transmission via CAT 5e(6) UTP cable

Up to 400m colour / 600m b&w (at unshielded cable)

Approx. 200m colour / 300m b&w (at shielded cable)

No adjustment necessary

Connectors: BNC plug / 2 pin terminal block

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Model | mini module |
| Type of unit | transmitter / receiver |
| Range | 400 m |
| Signal transmission | video |
| Protection rating | n/a |
| Operation mode | passive |
| Manufacturer order number | 75802 |

Additional Productinformation

Preferably unshielded twisted-pair cables should be used for the signal transmission. Are there already existing shielded cables which have to be used, the shield must only be earthed on one end, e. g. on transmitter side. We recommend cables in a twisted 2- pair wire bundle configuration (n2x2x0.8mm). The best result will be achieved by using cables with a twist of up to 7-times per meter.  Shielded cables, cables with different wire diam., or a connection via patch fields, can considerably reduce the max. possible transmission length.

Related products

Recommendations

Services