Specifications text

Article number

239837

Type

KBD-3NET

Short description

Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC

Main Features

Network controller with keyboard

Supports eneo network cameras

Functions PTZ, focus, zoom, preset, tours

Supports standard ONVIF protocol

Connection via RJ45 interface

RS-485 interface supports Pelco D/P protocol

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 140(150)x306x99mm |
| Joystick | 3-axis |
| Housing | plastic |
| Color | anthracite |
| Power consumption | 12W |
| Operating temperature | 0°C ~ +55°C |
| Weight | 1.3kg |
| Network connection | 1 x 10/100BASE-T |
| Manufacturer order number | 239837 |

Additional Productinformation

<p><span class="jCAhz ChMk0b"><span class="ryNqvb">The network system keyboard with joystick offers versatile application options for controlling and managing surveillance systems.</span></span> <span class="jCAhz ChMk0b"><span class="ryNqvb">Thanks to support for the ONVIF protocol, it is compatible with a wide range of IP cameras, including eneo network cameras, making it easier to integrate into existing surveillance networks.</span></span><span class="jCAhz ChMk0b"><span class="ryNqvb"> The connection is made via an RJ45 interface, which enables easy integration into networks.</span></span></p><p><span class="jCAhz ChMk0b"><span class="ryNqvb">In addition to network communication, the keyboard also supports classic serial interfaces, such as the RS-485 interface, to control cameras via the Pelco D/P protocol.</span></span> <span class="jCAhz ChMk0b"><span class="ryNqvb">This extends compatibility to a wide range of PTZ cameras that enable precise control of pan, tilt and zoom functions (PTZ).</span></span></p><p><span class="jCAhz ChMk0b"><span class="ryNqvb">The main functions of the keyboard include controlling PTZ cameras, focus and zoom settings, and calling up and controlling preset positions.</span></span> <span class="jCAhz ChMk0b"><span class="ryNqvb">The integrated joystick ensures intuitive and smooth operation, which is of great importance in safety-critical areas such as surveillance control rooms, traffic control centers or in large building complexes.</span></span> <span class="jCAhz ChMk0b"><span class="ryNqvb">The 12VDC power supply makes the keyboard flexible to use, as it is compatible with many standardized surveillance installations.</span></span></p><p><span class="jCAhz ChMk0b"><span class="ryNqvb">Overall, this network keyboard is ideal for professional use in video surveillance and offers a high level of flexibility thanks to its compatibility with ONVIF and support for multiple control protocols.</span></span></p>

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services