Specifications text

Article number

217871

Type

ISD-55P0010P0A

Short description

1/1.8" Network Dome, Fixed, Day&Night, 2592x1944, 360°, H.264, 1.05mm, Indoor

Main Features

1/1.8" Sony STARVIS CMOS-Sensor

1.05mm fixed lens FishEye camera

360° Panorama view

Integration in eneo Center Software

True Day&Night

Resolution max. 2048x1536@15fps

Compression H.264/MJPEG, Web Casting

Cross Web-Browsing

D-WDR (Digital Wide Dynamic Range)

Framerate increase on motion detection

OpenVPN support

Compact design for indoor application

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 63,8x119mm |
| Series | eneo SN-Series |
| Camera Type | Fisheye Kamera |
| Resolution Standard | 5 Megapixel |
| System | True Day&Night |
| Sensor size | 1/1.8" |
| Imager | CMOS |
| Video compression | H.264, M-JPEG |
| Image resolution max. | 2592x1944 |
| Image Transmission Rate | 15 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,3 Lux at F2,8 |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 1,05 mm |
| Digital Zoom | yes |
| Observe (62 px/m) | 14 m |
| Recognize (125 px/m) | 6 m |
| Identify (250 px/m) | 4 m |
| Detect (25px/m) | 36 m |
| Horizontal Angle of View | 360° |
| Vertical Angle of View | 360° |
| llumination | no |
| Housing | Indoor |
| Color (housing) | white |
| Hosuing material | Plastic |
| Heater | no |
| Blower | no |
| Alarm inputs | 1 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSDHC card, microSDXC card |
| Analysis functions | Defog, Tampering protection |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 5.3W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| ONVIF | ONVIF Profile S |
| Low Speed Shutter | no |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Camera type | Dome |
| Audio | 2 Way Audio |
| Remote configuration | Browser |
| Weight | 270g |
| Manufacturer order number | 217871 |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| IAD-5SG1008MUA | Gigabit Switch, Unmanaged, 120W, 8x PoE, 1xSFP, 1x RJ45, 10/100/1000 Mbps, Desktop |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |

Recommendations

| WDD064G1P0C | MicroSDXC Speicherkarte, 64GB, Class 10, UHS-I U1, 32TBW, für Videoüberwachung |
| --- | --- |

Services

| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| --- | --- |
| LACKIERUNG PTZ oder Panoramakamera | Extra charge for special paint PTZ dome or panoramic camera in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |