Specifications text

Article number

220201

Type

IPP-72A0023MIA

Short description

1/2,8" Network Dome, PTZ, Day&Night, 23x, H.265, WDR, 1920x1080, Infrared, IP66

Main Features

1/2.8" Sony STARVIS CMOS-Sensor

Full HD 1920x1080 Pixel, 25/30fps

23x AF Zoom, 5~117mm

Quad Stream H.265, H.264, MJPEG (Smart Codec)

Real time WDR (Wide Dynamic Range) 25/30fps

Intelligent self learning video analysis

Digital Noise Reduction (2DNR/3DNR)

Audio support including audio alarm

Infrared illumination up to 60m

Alarm in- and outputs, micro SD-card

Power supply 12VDC or HPoE

Support ONVIF Profile S und Profile T

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 208,3x182,4mm |
| Series | eneo SN-Series |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,35 Lux at F1,6, 50 IRE |
| Digital Noise Reduction | yes |
| Lens Type | auto focus zoom |
| Focal Length | 5 - 117 mm |
| Iris Control | DC |
| Zoom Factor | 23x |
| Digital Zoom | yes |
| Observe (62 px/m) | 27 - 590 m |
| Recognize (125 px/m) | 14 - 290 m |
| Identify (250 px/m) | 7 - 140 m |
| Detect (25px/m) | 71 - 1400 m |
| Horizontal Angle of View | 56,6 - 3° |
| Vertical Angle of View | 31,8 - 1,6° |
| MOD (Mininum Object Distance) | 3 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | black, white |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | yes |
| Rotation range | 360° |
| Speed (manual) | 380°/Sek. |
| Speed (preset) | 380°/Sek. |
| Video outputs | no |
| Alarm inputs | 4 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSDHC card, microSDXC card |
| Analysis functions | Cross Counting (CC), Defog, Digital image stabilization, Human (PD), Loitering detection, Motion detection, Perimeter Intrusion Detection (PID), Tampering Detection (TD) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 22W |
| Power over Ethernet | IEEE 802.3at |
| Vandal proof | no |
| Operating temperature | -20°C ~ +50°C |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | PTZ |
| Audio | 2 Way Audio |
| Remote configuration | Browser |
| Illumination range | approx. 60m |
| Weight | 2.8kg |
| Manufacturer order number | 220201 |

Additional Productinformation

Related products

| DOME-WB2-W | Wall bracket for IED-63xx Metal series and IEP-63Mxx, MEP-62Mxx |
| --- | --- |
| EDMC/CWB-1-W | Wall or Pendant Mount Kit white, for Dome Camera Series MPP-72xx, IPP-72xx |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| WDD064G1P0C | MicroSDXC Speicherkarte, 64GB, Class 10, UHS-I U1, 32TBW, für Videoüberwachung |
| --- | --- |

Services

| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| --- | --- |
| LACKIERUNG PTZ oder Panoramakamera | Extra charge for special paint PTZ dome or panoramic camera in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |