Specifications text

Article number

219526

Type

IAM-5SG1004MMA

Short description

Gigabit Switch, Managed, 120W, 4x PoE, 2x SFP, 1000Mbps, Industrial Design, -40°C~+75°C

Main Features

Gigabit PoE Switch, DIN Rail

2x SFP Gigabit Uplink Ports

4x 10/100/1000 PoE Ports, IEEE 802.3af/at

Supports VLAN, RTSP, "Fast Ring", and QoS

Up to 30W per port, Total ?120W

Fast Ring-Redundant, Reconfiguration <20ms

Supports redundant power supply connection

ESD protection IEC61000-4-5 Standard

Web configuration, Web management

Fault Relay, Console Port, Status LED

PoE reset button for each port

CCTV mode allows transmission up to 250m @ 10Mbps

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 46,5x110x163mm |
| Type | Layer2 switch |
| Series | eneo Switch |
| Mounting | DIN rail, wall mount |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | yes |
| Ports | 2x 1000Base-X SFP, 4x 10/100/1000Base-T |
| PoE ports (number) | 4 |
| Management | yes |
| Model | DIN rail module |
| Supply voltage | 48~57 VDC |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 120 W |
| Operating temperature | -40°C ~ +75°C |
| Weight | 680g |
| Manufacturer order number | 219526 |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

| jetpower-NDR-240-48 | Netzgerät für DIN Rail Montage 230VAC Eingang, 48VDC Ausgang max. 240W |
| --- | --- |

Services