Specifications text

Article number

217280

Type

MPC-52C0000M0A

Short description

1/2.8" HD Camera, Day&Night, 1920x1080, WDR, C/CS, 12/24V

Main Features

1/2.8" 2 Megapixel SONY STARVIS CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

C/CS mount, LSC (Lens shading compensation)

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 72x74x89mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo COAXIZE, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,08 Lux at F1,4, 40 IRE |
| Digital Noise Reduction | yes |
| Iris Control | DC |
| Housing | Indoor |
| Color (housing) | Pantone Cool Gray 1C |
| Hosuing material | Aluminium |
| Video outputs | AHD, Composite (CVBS), CVI, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 5W |
| Operating temperature | -10°C ~ +50°C |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Motion Detector | yes |
| Signal format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Box |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Weight | 290g |
| Manufacturer order number | 217280 |

Additional Productinformation

<h2>Full HD Kamera Multisignal</h2><p>Überwachungskameras mit Multisignaltechnologie empfehlen sich für alle Anwendungen, bei denen Echtzeit-Aufnahmen in HD bzw. Full-HD gefragt sind, die auf dem Monitor latenzfrei und ohne störende Artefakte dargestellt werden. Die eneo Full HD Kamera MPC-52C0000M0A (Box-Type) generiert wahlweise Videostreams in HD-TVI, AHD, CVI, EX-SDI, HD-SDI und Composite (FBAS). Das macht die solide Full HD Kamera im Aluminiumgehäuse ideal für das wirtschaftliche Upgrade von Bestandssystemen auf HD und Full-HD. Und dank des eneo typischen Plug-and-Play-Designs ist MPC-52C0000M0A errichterfreundlich und im Handumdrehen einsatzbereit. <br><br></p><h3>Herausragende Bildqualität</h3><p>Die eneo Full HD Kamera ist als True Day&Night-Modell mit einem 2 Megapixel Sony Starvis CMOS-Sensor ausgestattet. Geeignete Objektive finden sich bei eneo in großer Auswahl (C/CS Mount). BLC/HLC Gegenlichtkompensation und Wide Dynamic Range optimieren automatisch die Bildqualität der Full HD Kamera auch bei schwierigen Lichtverhältnissen. Bei Smog, Nebel oder Nieselregen sorgt die Defog-Funktion der kompakten Box-Kamera für klare Bilder. Für den Einsatz in Kombination mit IR-Scheinwerfern ist die Full HD Kamera mit einem schwenkbaren IR-Sperrfilter ausgestattet. <br><br></p><h3>Schutz der Privatsphäre und Bewegungserkennung</h3><p>Seit Inkrafttreten der EU-DSGVO genießt der Schutz personenbezogener Daten im Rahmen der Videoüberwachung besonders hohe Priorität. Die eneo Full HD Kamera mit Multisignalunterstützung ermöglicht die individuelle Schwärzung von Bildbereichen mittels Privatzonenmaskierung. Außerdem verfügt sie über einen integrierten Bewegungsmelder, der bei Aktivität im Überwachungsbereich einen Alarm auslöst. </p>

Related products

| H04Z03NFDC-MP | F1,5/4-13mm, Megapixel DC-Lens, variable Focal Lenght, 1/1.8" CS-Mount |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| WD-30 | Wall Mount Bracket with Ball Joint, 1/4" Camera Fixing Screw, Pantone Cool Grey 1C |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |