Specifications text

Article number

217278

Type

MPD-72M2713M0A

Short description

1/2.8" HD Dome, Fixed, WDR, Day&Night, 1920x1080, 12/24V 2.7-13.5mm, Infrared, IP67

Main Features

1/2.8" 2 Megapixel SONY STARVIS CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized vari-focal lens 2.7-13.5mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

IP67, built-in heater, 3-Axis gimbal

For wall or ceiling flush mount

Hard coated bubble, Anti-scratch protection

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 119x140mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo Callisto, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,08 Lux at F1,4, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal (motorized) |
| Focal Length | 2,7 - 13,5 mm |
| Iris Control | DC |
| Zoom Factor | 4x |
| Digital Zoom | no |
| Observe (62 px/m) | 18 - 52 m |
| Recognize (125 px/m) | 8 - 25 m |
| Identify (250 px/m) | 4 - 12 m |
| Detect (25px/m) | 47 - 128 m |
| Horizontal Angle of View | 100 - 38° |
| Vertical Angle of View | 53 - 19° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone Cool Gray 1C |
| Color (bubble) | clear |
| Hosuing material | Aluminium |
| Heater | yes |
| Rotation range | +/-173° |
| Video outputs | AHD, Composite (CVBS), CVI, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 9.5W |
| Vandal proof | yes |
| Operating temperature | -30°C ~ +50°C |
| Certifications | IK10 |
| Protection rating | IP67 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Motion Detector | yes |
| Signal format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Dome |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.4 |
| Illumination range | approx. 50m |
| Weight | 1 kg |
| Manufacturer order number | 217278 |

Additional Productinformation

<h2>Modularer eneo HD Dome für den Wand- oder Deckeneinbau </h2><p>Der HD Dome MPD-72M2812M0A von eneo bietet Betreibern und Anwendern viel Flexibilität für die reibungslose Integration in bestehende Videosicherheitslösungen. Möglich machen es die diversen Videosignale, die der HD Dome unterstützt: Anwender können unter den analogen Videoformaten HD-TVI, AHD, CVI und Composite sowie den digitalen Signaltypen EX-SDI, HD-SDI das für ihr jeweiliges Bestandssystem geeignete Format auswählen. Hierfür ist der MPD-72M2812M0A mit separaten Ausgängen für die digitalen und analogen Signale ausgestattet. Das Objektiv wird durch eine kratzfeste Kuppel geschützt und zusätzlich ist für den eneo HD Dome eine breite Palette an praktischem Installationszubehör erhältlich, mit dem sich die Kamera im Handumdrehen professionell und optisch ansprechend an Wand, Decke oder Masten montieren lässt. Hierfür bietet eneo Errichtern geeignete Montageringe, Keiladapter, Wandarme oder Anschlussboxen. Der 3-Achsen-Aufhängering sorgt für zusätzlichen Komfort bei der Montage. <br><br></p><h3>Motorisiertes Varifokalobjektiv und integrierte IR-Beleuchtung </h3><p>Der eneo HD Dome kommt mit einem motorisierten Varifokalobjektiv (Brennweite 2,8?12mm) und einem Sony Starvis CMOS-Sensor. Die maximale Videoauflösung beträgt 1920x1080 Pixel (Full HD). Für den Nachtbetrieb verfügt die Kamera über zwei integrierte High-Power-LEDs (850nm, Reichweite ca. 50m) und einen IR-Sperrfilter, der bei Dunkelheit wegschwenkt, so dass das für Nachtaufnahmen nötige Infrarotlicht auf den Sensor trifft. Damit bietet der HD Dome Videosicherheit rund um die Uhr. <br><br></p><h3>Bildoptimierungsfunktionen für konstante Leistung </h3><p>Damit Anwender auch bei schwierigen Beleuchtungsbedingungen oder schlechtem Wetter die Lage im Überwachungsbereich gut überblicken können, ist der eneo HD Dome mit den Bildoptimierungsfunktionen WDR, BLC / HLC Gegenlichtkompensation und Defog (automatische Bildkorrektur bei Nebel, Nieselregen oder Smog) ausgestattet. In Tests wurden für den HD Dome die folgenden DORI-Distanzen ermittelt: Wahrnehmen (25 px/m) auf 47 bis 128m, Beobachten (62 px/m) auf 18 bis 52m, Erkennen (125 px/m) auf 8 bis 25m und Identifizieren (250 px/m) auf 4 bis 12m. <br><br></p><h3>Robustes Design für den Außeneinsatz </h3><p>MPD-72M2812M0A kommt mit einem soliden, IP67-konformen Aluminiumgehäuse, das der Schutzklasse IK10 entspricht und damit auch gegen Vandalismus geschützt ist. Diese Eigenschaften sowie die integrierte Heizung, die den Betrieb bei Temperaturen von -30°C?50°C ermöglicht, prädestinieren den eneo HD Dome für den Tag-und-Nachteinsatz im Außenbereich.</p>

Related products

| AK-5 | Camera Junction Box, for Callisto Dome series, IP66 |
| --- | --- |
| DOME-S1 | Sun Shield for eneo Callisto Dome Series |
| DOME-WB1+DOME/ Basis1 | Wall Bracket & Surface mount base, Callisto Dome line |
| DOME-WB3 | Wall bracket for eneo Dome Cameras MED-62xxxxxM0A and IED-62xxxxxM0A |
| DOME/BASIS1 | Surface mount base , Callisto Dome line |
| DOME/WEDGE2 | Wedge Adapter, Callisto Dome Line |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-IPM07PRO | 7" (17,8cm) Test Monitor, Touch, IP, HD/3G/EX-SDI, TVI, CVI, AHD, Composite, HDMI |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |