Specifications text

Article number

211467

Type

HDD-980MIR B

Short description

1/3" HD-SDI Dome, WDR, Day&Night, 1920x1080, 3.6mm, Infrared, 12V

Main Features

1/3" 2.0 MP progressive scan CMOS sensor

Resolution max. 1920x1080 (Full HD)

Support HD-SDI and EX-SDI format

3.6mm Mega Pixel lens

Removable infrared cut filter

IR illumination

WDR, 2&3DNR Noise Reduction, Defog function

Integral motion detection

1.5G HD-SDI video transmission via coax

Wall or ceiling mount

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | see drawing |
| Series | eneo COAXIZE |
| Resolution Pixels | 1280x720, 1920x1080 |
| System | True Day&Night |
| Sensor size | 1/3" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,5 Lux (Color) 0,1Lux (BW) |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 3,6 mm |
| Horizontal Angle of View | 86° |
| llumination | Infrared |
| Housing | Indoor |
| Color (housing) | white |
| Color (bubble) | clear |
| Hosuing material | Plastic |
| Heater | no |
| Video outputs | EX-SDI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | no |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 5W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| Protection rating | n/a |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR |
| Motion Detector | yes |
| Signal format | EX-SDI (270 Mbps), HD-SDI (1.485 Gbps) |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F2.0 |
| Weight | 190g |
| Manufacturer order number | 211467 |

Additional Productinformation

Related products

| VT-PS12DC-11 | Stecker Netzgerät, 100-240VAC/12VDC (1,0A), stabilisiert, mit Hohlstecker |
| --- | --- |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-IPM07HD | Test Monitor, Multiformat, for installation, IP, HD-TVI/CVI, HD-AHD, HD-SDI, CVBS, HDMI |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services