Specifications text

Article number

217349

Type

IER-38R320005A

Short description

Network Video Recorder, 32x IP, H.265, H.264, w/o HDD, HDMI, 16x PoE

Main Features

32-channel network video recorder

Easy installation with eneo IP cameras

Compression Method: H.264, H.265

Resolution IP cameras up to 8 megapixels

Remote Access: Control of additional recorders

Memory expandable via iSCSI and eSATA

CMS administration software included

Smartphone app for video monitoring / configuration

16x alarm input and 1x alarm output,POS interface

16x PoE port (IEEE802.3af), total power 130W

Supports dewarping for (ISD-55xx/ISD-59xx)

1x HDMI monitor output with 4K UHD resolution

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Manufacturer order number | 217349 |

Additional Productinformation

Related products

| eneo Center | Remote Software IER-25xx, IER-28xx, IER-38xx, MSR-24xx, MSR-25xx, MER-Serie |
| --- | --- |
| eneo Center mobile | Mobile App for IER-25xx, IER- IER-28xx. IER-38xx, IEM-38xx MSR-24xx, MSR-25xx |
| eneo Site Manager | Management Software for eneo IP products |
| HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |
| HDD-8000SATA Purple | Hard Disk Drive 8TB, SATA, for eneo Video Recorders, expansion |
| IEM-38R640005A | 64-Channel Virtual Matrix for IER-, MSR-Series, H.264, H.265 640Mbps, w/o HDD, HDMI |
| KBD-2USB | System Keyboard with 3-Axis Joystick, Jog Shuttle, USB, RS-485, 12VDC, |

Recommendations

| WD Ultrastar SATA 10TB | Festplatte, 3,5", SATA 6Gb/s, 10TB, 7200RPM, 256MB, 24/7, R/V Sensor |
| --- | --- |

Services

| CON-eneo-KON-04 | Preconfiguration 1x eneo recorder with up to 4 eneo IP cameras |
| --- | --- |
| CON-eneo-KON-08 | Preconfiguration 1x eneo recorder1x eneo IE with up to 8 eneo IP cameras |
| CON-eneo-KON-16 | Preconfiguration 1x eneo recorder with up to 16 eneo IP cameras |
| CON-eneo-KON-32 | Preconfiguration 1x eneo recorder with up to 32 eneo IP cameras |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |
| IER/PNR-KON16 | Configuration 1x PNR-53xx with 1-16 eneo IP Cameras |
| IER/PNR-KON32 | Configuration 1x PNR-53xx with 1-32 eneo IP Cameras |
| IER/PNR-KON4 | Configuration 1x PNR-53xx with 1-4 eneo IP Cameras |